FOG Spill‑Response Standard Operating Procedure (SOP)

Version: ____ Effective Date: ____

1 Purpose

Provide clear, immediate steps to stop, contain, absorb, and report any kitchen spill of fats, oils, or grease (FOG), preventing drain entry, slip hazards, and regulatory fines.

2 Scope

Applies to all employees at [Facility Name] and covers spills up to 25 gallons contained indoors. Spills reaching drains or outside areas trigger SPCC emergency protocols.

3 Responsibilities

| Role | Task |
| --- | --- |
| Any Employee | Initiate spill kit, notify Spill Coordinator. |
| Spill Coordinator | Direct cleanup, contact external agencies if needed, file incident log. |
| BOH Manager | Restock spill kit within 24 hours; upload incident report to dashboard. |

4 Procedure

- Stop the source – Close valve, upright container, or shut pump if safe to do so.

- Protect drains – Place drain covers or absorbent booms over nearby floor drains.

- Contain the pool – Circle the spill with absorbent pads and additional booms.

- Absorb and collect – Use granular absorbent for remaining liquid; scoop saturated media into yellow spill bin.

- Clean and degrease – Mop with degreaser, rinse, and dry floor to remove slip hazard.

- Report – Fill out Spill Incident Log (Section 5) and attach timestamped photo.

- Notify – If spill reaches a drain or exceeds 25 gal, call the National Response Center (1‑800‑424‑8802) within 15 minutes and local authorities as required.

5 Records

- Spill Incident Log – retain 3 years.

- Photo evidence – retain 3 years with log.

- NRC report number (if filed) – attach to SPCC plan.

6 References

- SPCC Plan Template, Section 3 – Emergency Contacts & Response

- OSHA 29 CFR 1910.1200 – Hazard Communication

- County FOG Ordinance spill‑cleanup requirements